For each scalp-mesh point, PCA is applied not to the whole head, but to a small local neighborhood around that point.

Let the mesh point of interest be

p0=x0y0z0.

Choose Nneighboring surface points:

pi=xiyizi,i=1,…,N.

These can be the nearest mesh vertices or all vertices within a selected radius.

1. Construct the raw data matrix

Place the coordinates of the neighboring points in rows:

P=x1y1z1x2y2z2⋮⋮⋮xNyNzN.

Thus:

- each row represents one neighboring scalp point;

- the three columns are the x, y, and zcoordinates;

- the matrix has dimensions N×3.

You may include p0itself in the neighborhood, although this is not essential.

2. Center the coordinates

PCA must be performed on coordinates relative to the local neighborhood center, not on the original MRI coordinates.

Calculate the centroid:

p=1Ni=1Npi=xyz.

Then subtract the centroid from every row:

X=x1-xy1-yz1-zx2-xy2-yz2-z⋮⋮⋮xN-xyN-yzN-z.

This centered N×3matrix Xis the actual PCA data matrix.

3. Construct the covariance matrix

The local covariance matrix is

C=1N-1XTX.

Because Xhas three coordinate columns, Cis a 3×3matrix:

C=CxxCxyCxzCyxCyyCyzCzxCzyCzz.

This matrix describes how the neighboring surface points vary in three-dimensional space.

4. Calculate eigenvalues and eigenvectors

Solve

Cvj=λjvj.

Sort the eigenvalues:

λ1≥λ2≥λ3.

The corresponding eigenvectors are

v1,v2,v3.

Their geometric meanings are:

- v1: direction of greatest variation along the surface;

- v2: second direction along the surface;

- v3: direction of least variation.

Because the points lie approximately on a local surface patch, they vary substantially in two tangential directions but very little perpendicular to the surface. Therefore,

n=v3

is the estimated local surface normal.

In other words, the eigenvector corresponding to the smallest eigenvalue is the normal.

Conceptually, PCA fits the plane that best approximates the local points. The normal is perpendicular to that plane.

5. Determine whether the normal points outward

PCA provides the normal direction only up to sign:

nand-n

are mathematically equivalent. You must therefore orient it outward.

A simple approach is to define an approximate interior point of the head, such as the centroid of the full scalp surface:

chead.

Then calculate

r=p0-chead.

If

n⋅r<0,

reverse the normal:

n←-n.

After this operation, npoints approximately away from the head.

For a generally convex scalp surface, this works well. Around strongly concave regions, a mesh-based orientation method may be more reliable.

6. Generate the ESE point

Once the outward unit normal is known, calculate

pESE=p0+dn,

where dis the distance from the MRI scalp surface to the chosen electrode external reference surface.

For example, if

p0=204080 mm,n=0.20.30.9327,

and the offset is d=5mm, then

pESE=204080+50.20.30.9327=2141.584.6635 mm.

A small numerical illustration

Suppose the neighborhood contains six points:

P=-1-110.10-110.01-19.9-1110.00110.11110.0.

The points vary strongly in xand y, but only slightly in z. After centering and PCA:

- the two largest eigenvectors will lie approximately in the xy-plane;

- the smallest eigenvector will be approximately

v3≈001.

Therefore, the local surface normal is approximately vertical.

Choosing the neighborhood

This is important for VIRDA because the scalp mesh can contain MRI segmentation noise.

A very small neighborhood gives a highly local estimate but may produce noisy normals. A very large neighborhood gives smooth normals but may overlook genuine local curvature.

Two practical methods are:

Fixed number of neighbors

For each vertex, take the knearest mesh vertices, for example:

k=20, 30, or 50.

This is convenient when mesh density is relatively uniform.

Fixed physical radius

Take all points within a radius such as:

r=5–10 mm.

This is often preferable because it corresponds to a real anatomical scale. For VIRDA, it may be useful to test several scales, including a radius related to the electrode dimensions.

I would initially test perhaps 5, 8, and 10mm neighborhoods and visually compare the resulting normal fields.

Weighted PCA  (OPTIONAL)

A useful refinement is to give nearby points more influence than distant points. Define a weight, for example,

wi=exp⁡∥pi-p0∥22σ2.

Then use the weighted centroid

pw=iwipiiwi,

and weighted covariance

Cw=1iwiiwi(pi-pw)(pi-pw)T.

The eigenvector of Cwwith the smallest eigenvalue is again the normal. Weighted PCA can reduce abrupt changes caused by the boundary of the selected neighborhood.

One additional quality measure

The eigenvalues can tell you how reliable the estimated normal is.

For a good locally planar surface patch,

λ3≪λ1,λ2.

A useful measure is

q=λ3λ1+λ2+λ3.

A small qindicates that the points lie close to a plane and the normal estimate is reliable. A larger qmay indicate:

- segmentation noise;

- a sharp anatomical feature;

- an unsuitable neighborhood size;

- insufficient mesh density.

Thus, PCA provides not only a normal vector but also a local estimate of confidence.